การใช้งาน ThingsBoard IoTs Platform เพื่อสร้างและจัดการระบบอัฉริยะ ThingsBoard IoTs Platform for smart system

ขื่อ-สกุล : นายธนพล กาศักดิ์

6/6 -- คำถามท้ายบทเพื่อทดสอบความเข้าใจ

Quiz_101 – ThingsBoard Data Monitor

- Mission - 1/4: ให้ส่งข้อมูลค่า Humidity และ Temperatures จากเซ็นเซอร์ DHT-22 ไปยัง Dashboard

- #include "ThingsBoard.h" - #include "DHTesp.h" - #include <WiFi.h> - #define WIFI_AP "JAEWHON_2.4G" //Your Wifi - #define WIFI_PASSWORD "4144284312" //Your Wifi password - #define TOKEN "bp8fF8oofRkDx2EXU5eL" - #define THINGSBOARD_SERVER "demo.thingsboard.io" - #define Pin_DHT22 15 - #define SERIAL_DEBUG_BAUD 115200 - WiFiClient espClient; - DHTesp dht; - ThingsBoard tb(espClient); - int status = WL_IDLE_STATUS; - void setup() { - // initialize serial for debugging - Serial.begin(SERIAL_DEBUG_BAUD); - dht.setup(Pin_DHT22, DHTesp::DHT22); - WiFi.begin(WIFI_AP, WIFI_PASSWORD); - InitWiFi(); - } - void loop() { - if (WiFi.status() != WL_CONNECTED) { - reconnect(); - } - if (!tb.connected()) { - // Connect to the ThingsBoard - Serial.print("Connecting to: "); - Serial.print(THINGSBOARD_SERVER); - Serial.print(" with token "); - Serial.println(TOKEN); - if (!tb.connect(THINGSBOARD_SERVER, TOKEN)) { - Serial.println("Failed to connect"); - return; - } - } - Serial.print("Sending data...");float xTempp = dht.getTemperature(); - float xHdmid = dht.getHumidity(); - Serial.print(xTempp, 2); - Serial.print(","); - Serial.print(xHdmid, 2); - Serial.println();tb.sendTelemetryFloat("temperature", xTempp); - tb.sendTelemetryFloat("humidity", xHdmid); - tb.loop(); - delay(5000); - } - void InitWiFi() { - Serial.println("Connecting to AP ..."); - // attempt to connect to WiFi network - WiFi.begin(WIFI_AP, WIFI_PASSWORD); - while (WiFi.status() != WL_CONNECTED) { - delay(500); - Serial.print("."); } - Serial.println("Connected to AP"); - } - void reconnect() { - // Loop until we're reconnected - status = WiFi.status(); - if ( status != WL_CONNECTED) { - WiFi.begin(WIFI_AP, WIFI_PASSWORD); - while (WiFi.status() != WL_CONNECTED) { - delay(500); - Serial.print("."); - } - Serial.println("Connected to AP"); } - }

Capture Dashboard

รูปการทดสอบ 1

รูปการทดสอบ 2

Quiz_102 – ThingsBoard Data Monitor and Control

- Mission 2/4: ให้ส่งข้อมูลค่า Humidity และ Temperatures จากเซ็นเซอร์ DHT-22 ไปยัง ThingsBoard พร้อมทั้งควบคุม On/Off - 4 LED และ Blink Speed สำหรับอีก 1 LED

โปรแกรมที่ใช้ทดสอบ #define COUNT_OF(x) ((sizeof(x)/sizeof(0[x])) / ((size_t)(!(sizeof(x) % sizeof(0[x]))))) #include <WiFi.h> #include <ThingsBoard.h> #include "DHTesp.h" #define WIFI_AP_NAME "JAEWHON_2.4G" //Your Wifi #define WIFI_PASSWORD "4144284312" //Your Wifi password #define TOKEN "bp8fF8oofRkDx2EXU5eL" #define THINGSBOARD_SERVER "thingsboard.cloud" #define pinLEDBlink 2 #define Pin_DHT22 15 WiFiClient espClient; DHTesp dht; ThingsBoard tb(espClient); int status = WL_IDLE_STATUS; uint8_t leds_PinControl[] = {19, 21, 22, 23}; int leds_Ststus[] = { 0, 0, 0, 0 }; char StringEcho[] = "stsLED_1"; int loopDelay = 20; // Main loop delay(ms) int sendDataDelay = 2000; // Period of Sending Tempp/Humid. int BlinkLEDDelay = 500; // Initial period of LED cycling. int Count_BlinkLEDDelay = 0; // Time Counter Blink peroid int Count_sendDataDelay = 0; // Time Counter Sending Tempp/Humid bool Subscribed_Status = false; // Subscribed_Status for the RPC messages. int ststus_BlinkLED = 0; // LED number that is currenlty ON. #include "_ThingBoardRPC.h" #include "_ConnectWifi.h" //===================================================== void setup() { // Initialize serial for debugging Serial.begin(115200); WiFi.begin(WIFI_AP_NAME, WIFI_PASSWORD); WiFi_Initial(); // Pinconfig dht.setup(Pin_DHT22, DHTesp::DHT22); pinMode(pinLEDBlink, OUTPUT); for (size_t i = 0; i < COUNT_OF(leds_PinControl); ++i) { pinMode(leds_PinControl[i], OUTPUT); } } //===================================================== void loop() { // Step0/6 - Loop Delay delay(loopDelay); Count_BlinkLEDDelay += loopDelay; Count_sendDataDelay += loopDelay; // Step1/6 - Check if next LED Blink if (Count_BlinkLEDDelay > BlinkLEDDelay) { digitalWrite(pinLEDBlink, ststus_BlinkLED); ststus_BlinkLED = 1 - ststus_BlinkLED; Count_BlinkLEDDelay = 0; } // Step 2/6 - Reconnect to WiFi, if needed if (WiFi.status() != WL_CONNECTED) { reconnect(); return; } // Step 3/6 - Reconnect to ThingsBoard, if needed if (!tb.connected()) { Subscribed_Status = false; // Connect to the ThingsBoard Serial.print("Connecting to: "); Serial.print(THINGSBOARD_SERVER); Serial.print(" with token "); Serial.println(TOKEN); if (!tb.connect(THINGSBOARD_SERVER, TOKEN)) { Serial.println("Failed to connect"); return; } } // Step 4/6 - Subscribe for RPC, if needed if (!Subscribed_Status) { Serial.println("Subscribing for RPC..."); // Perform a subscription. All consequent data processing will happen in // callbacks as denoted by callbacks[] array. if (!tb.RPC_Subscribe(callbacks, COUNT_OF(callbacks))) { Serial.println("Failed to subscribe for RPC"); return; } Serial.println("Subscribe done"); Subscribed_Status = true; } // Step 5/6 - Check if it is a time to send Tempp/Humid if (Count_sendDataDelay > sendDataDelay) { Serial.print("Sending data..."); float temperature = dht.getTemperature(); float humidity = dht.getHumidity(); tb.sendTelemetryFloat("temperature", temperature); tb.sendTelemetryFloat("humidity", humidity); Serial.print("T=" + String(temperature, 2) + ", "); Serial.print("H=" + String(humidity, 2) + ", "); Serial.print("LED="); for (size_t i = 0; i < COUNT_OF(leds_PinControl); ++i) { StringEcho[7] = 0x30 + i; // Set 0 to "0" tb.sendTelemetryInt(StringEcho, leds_Ststus[i]); Serial.print(leds_Ststus[i]); } Serial.println(); Count_sendDataDelay = 0; } // Step 6/6 - Process messages tb.loop(); } Wifi.h // _ConnectWifi.h //===================================================== void WiFi_Initial() { Serial.println("Connecting to AP ..."); // attempt to connect to WiFi network WiFi.begin(WIFI_AP_NAME, WIFI_PASSWORD); while (WiFi.status() != WL_CONNECTED) { delay(500); Serial.print("."); } Serial.println("\nConnected to AP"); Serial.print("Local IP = "); Serial.println(WiFi.localIP()); } //===================================================== void reconnect() { status = WiFi.status(); // Loop until we're reconnected if ( status != WL_CONNECTED) { WiFi.begin(WIFI_AP_NAME, WIFI_PASSWORD); while (WiFi.status() != WL_CONNECTED) { delay(500); Serial.print("."); } Serial.println("\nConnected to AP"); Serial.print("Local IP = "); Serial.println(WiFi.localIP()); } } RPC.h // _ThingBoardRPC.h //########################################################### // Processes function for RPC call "setValue" // RPC_Data is a JSON variant, that can be queried using operator[] // See https://arduinojson.org/v5/api/jsonvariant/subscript/ for more details //========================================================== RPC_Response processDelayChange(const RPC_Data &data) { Serial.println("Received the set delay RPC method"); BlinkLEDDelay = data; Serial.print("Set new delay: "); Serial.println(BlinkLEDDelay); return RPC_Response(NULL, BlinkLEDDelay); } //########################################################### // Processes function for RPC call "getValue" // RPC_Data is a JSON variant, that can be queried using operator[] // See https://arduinojson.org/v5/api/jsonvariant/subscript/ for more details //========================================================== RPC_Response processGetDelay(const RPC_Data &data) { Serial.println("Received the get value method"); return RPC_Response(NULL, BlinkLEDDelay); } //########################################################### // Processes function for RPC call "setGpioStatus" // RPC_Data is a JSON variant, that can be queried using operator[] // See https://arduinojson.org/v5/api/jsonvariant/subscript/ for more details //========================================================== RPC_Response processSetGpioState(const RPC_Data &data) { Serial.println("Received the set GPIO RPC method"); int pin = data["pin"]; bool enabled = data["enabled"]; if (pin < COUNT_OF(leds_PinControl)) { Serial.print("Setting LED "); Serial.print(pin); Serial.print(" to state "); Serial.println(leds_Ststus[pin]); leds_Ststus[pin] = 1 - leds_Ststus[pin]; digitalWrite(leds_PinControl[pin], leds_Ststus[pin]); } return RPC_Response(data["pin"], (bool)data["enabled"]); } //########################################################### // RPC handlers //========================================================== RPC_Callback callbacks[] = { { "setValue", processDelayChange }, { "getValue", processGetDelay }, { "setGpioStatus", processSetGpioState }, };

Capture Dashboard

รูปถ่ายหน้า Web Broswer

รูปการทดสอบ 1

รูปการทดสอบ 2

Quiz_103 – ThingsBoard Data Monitor and control with MQTT Protocol

- Mission 3/4: ให้ใช้ MQTT กับ ThingsBoard

  - ปรับปรุงเพื่อให้ทำงานควบคุมการ On/Off - 4 LED

  - เพิ่มเติม คือ ทดสอบส่งข้อมูล 1 ค่าแบบสุ่มระหว่าง 00 – 50 ไปแสดงที่ Dashboard ด้วย ได้หรือไม่ ทำอย่างไรบ้างให้อธิบาย {Read https://thingsboard.io/docs/user-guide/device-profiles/ }

โปรแกรมที่ใช้ทดสอบ //Arduino Code // https://thingsboard.io/docs/samples/esp8266/gpio/ // https://blog.thingsboard.io/2017/01/esp8266-gpio-control-over-mqtt-using.html #include <WiFi.h> #include <ArduinoJson.h> // by Benoit Blanchon >> Ver 5.8.0 #include <PubSubClient.h> // by Nick O’Leary. >> Ver 2.6 and Update PubSubClient.h#define WIFI_AP_NAME "TRUE-BFFM 2G" #define WIFI_PASSWORD "7211722142622" #define Device_Name "new Device 2" #define Device_Token "HM35zge611h6BIMn78Ia" #define thingsboardServer "thingsboard.cloud"#define GPIO1_ESP32Pin 19 #define GPIO2_ESP32Pin 21 #define GPIO3_ESP32Pin 22 #define GPIO4_ESP32Pin 23 #define varTemp "temperature" float temperature; boolean gpioState[] = {false, false, false, false}; long lastMsgTime = 0; int status = WL_IDLE_STATUS;WiFiClient wifiClient; PubSubClient client(wifiClient);#include "_HandOnMQTT.h" #include "_WifiConnect.h"void setup() { Serial.begin(115200); // Set output mode for all GPIO pins pinMode(GPIO1_ESP32Pin, OUTPUT); pinMode(GPIO2_ESP32Pin, OUTPUT); pinMode(GPIO3_ESP32Pin, OUTPUT); pinMode(GPIO4_ESP32Pin, OUTPUT); delay(10); InitialWiFi(); client.setServer( thingsboardServer, 1883 ); client.setCallback(on_message); }void loop() { if ( !client.connected() ) { reconnect(); } sendTemperature(); client.loop(); }void sendTemperature() { if (millis() - lastMsgTime > 5000) { lastMsgTime = millis(); temperature = random(0, 50);StaticJsonBuffer<200> jsonBuffer; JsonObject & data = jsonBuffer.createObject(); data[String(varTemp)] = temperature; char payload[256]; data.printTo(payload, sizeof(payload)); String strPayload = String(payload); Serial.print("Get GPIO Status: "); Serial.println(strPayload); client.publish("v1/devices/me/telemetry", strPayload.c_str()); } } MQTT.h //======================================================== String get_gpio_status() { // Prepare gpios JSON payload string StaticJsonBuffer<200> jsonBuffer; JsonObject & data = jsonBuffer.createObject(); data[String(GPIO1_ESP32Pin)] = gpioState[0]; data[String(GPIO2_ESP32Pin)] = gpioState[1]; data[String(GPIO3_ESP32Pin)] = gpioState[2]; data[String(GPIO4_ESP32Pin)] = gpioState[3]; char payload[256]; data.printTo(payload, sizeof(payload)); String strPayload = String(payload); Serial.print("Get GPIO Status: "); Serial.println(strPayload); return strPayload; }//======================================================== void set_gpio_status(int pin, boolean enabled) { if (pin == GPIO1_ESP32Pin) { gpioState[0] = 1 - gpioState[0]; digitalWrite(GPIO1_ESP32Pin, gpioState[0]); } if (pin == GPIO2_ESP32Pin) { gpioState[1] = 1 - gpioState[1]; digitalWrite(GPIO2_ESP32Pin, gpioState[1]); } if (pin == GPIO3_ESP32Pin) { gpioState[2] = 1 - gpioState[2]; digitalWrite(GPIO3_ESP32Pin, gpioState[2]); } if (pin == GPIO4_ESP32Pin) { gpioState[3] = 1 - gpioState[3]; digitalWrite(GPIO4_ESP32Pin, gpioState[3]); } }//======================================================== // The callback for when a PUBLISH message is received from the server. void on_message(const char* topic, byte* payload, unsigned int length) { Serial.println("\nOn message"); char json[length + 1]; strncpy (json, (char*)payload, length); json[length] = '\0'; Serial.print("Topic: "); Serial.println(topic); Serial.print("Message: "); Serial.println(json); // Decode JSON request StaticJsonBuffer<200> jsonBuffer; JsonObject& data = jsonBuffer.parseObject((char*)json); if (!data.success()) { Serial.println("parseObject() failed"); return; } // Check request method String methodName = String((const char*)data["method"]);\ // If Reply with GPIO status if (methodName.equals("getGpioStatus")) { String responseTopic = String(topic); responseTopic.replace("request", "response"); client.publish(responseTopic.c_str(), get_gpio_status().c_str()); }// If Update GPIO status and reply if (methodName.equals("setGpioStatus")) { set_gpio_status(data["params"]["pin"], data["params"]["enabled"]); String responseTopic = String(topic); responseTopic.replace("request", "response"); client.publish(responseTopic.c_str(), get_gpio_status().c_str()); client.publish("v1/devices/me/attributes", get_gpio_status().c_str()); } } Wifi.h //======================================================== void InitialWiFi() { Serial.println("Connecting to AP ..."); WiFi.begin(WIFI_AP_NAME, WIFI_PASSWORD); while (WiFi.status() != WL_CONNECTED) { delay(500); Serial.print("."); } Serial.println("Connected to AP"); }//======================================================== void reconnect() { // Loop until we're reconnected while (!client.connected()) { status = WiFi.status(); if ( status != WL_CONNECTED) { InitialWiFi(); } Serial.print("Connecting to ThingsBoard node ..."); // Attempt to connect (clientId, username, password) if ( client.connect(Device_Name, Device_Token, NULL) ) { Serial.println( "[DONE]" ); // Subscribing to receive RPC requests client.subscribe("v1/devices/me/rpc/request/+"); // Sending current GPIO status Serial.println("Sending current GPIO status ..."); client.publish("v1/devices/me/attributes", get_gpio_status().c_str()); } else { Serial.print( "[FAILED] [ rc = " ); Serial.print( client.state() ); Serial.println( " : retrying in 5 seconds]" ); delay( 5000 ); // Wait 5 seconds before retrying } } }

Capture Dashboard

รูปถ่ายหน้า Web Broswer

รูปการทดสอบ 1

รูปการทดสอบ 2

Quiz_104 – Web Control 4 LED and Monitor Humid/Temperature

- Mission 4/4: การตรวจสอบและควบคุม อุณหภูมิ-ความชื้น ของโรงเรือนเลี้ยงไก่

  - ให้ใช้ ESP32 ส่งข้อมูลแบบสุ่มสองจำนวน คือ

    - Tempp_A	สุ่มระหว่าง 20-40

    - Hudmid_A	สุ่มระหว่าง 60-80

  - ข้อมูลทั้งสองค่าจะนำมาแสดงที่ Dashboard

  - สร้าง Alarm โดย หาก Tempp_A > 35 หรือ Hudmid_A > 70 ให้ Alarm

  - ศึกษาการตั้ง Alarm - https://thingsboard.io/docs/user-guide/alarms/

  - กำหนดรอบการตรวจสอบทุกๆ 20 วินาที

  - แชร์ Dashboard ไปให้ผู้ใช้งาน

โปรแกรมที่ใช้ทดสอบ #include "ThingsBoard.h" #include <WiFi.h>#define WIFI_AP "TRUE-BFFM 2G" #define WIFI_PASSWORD "7211722142622" #define TOKEN "z5dfxG3pUlssi9shOjzy" #define THINGSBOARD_SERVER "thingsboard.cloud"WiFiClient espClient; ThingsBoard tb(espClient); int status = WL_IDLE_STATUS;void setup() { Serial.begin(115200); WiFi.begin(WIFI_AP, WIFI_PASSWORD); InitWiFi(); }void loop() { if (WiFi.status() != WL_CONNECTED) { reconnect(); } if (!tb.connected()) { // Connect to the ThingsBoard Serial.print("Connecting to: "); Serial.print(THINGSBOARD_SERVER); Serial.print(" with token "); Serial.println(TOKEN); if (!tb.connect(THINGSBOARD_SERVER, TOKEN)) { Serial.println("Failed to connect"); return; } } Serial.println("\nSending data..."); float Tempp_A = random(2000,4000)/100.0; float Humid_A = random(6000,8000)/100.0; Serial.print("Tempp = " + String(Tempp_A, 2) + "'C"); Serial.print(","); Serial.println("Humid = " + String(Humid_A, 2) + "%"); tb.sendTelemetryFloat("Tempp_A", Tempp_A); tb.sendTelemetryFloat("Humid_A", Humid_A); tb.loop(); delay(5000); }void InitWiFi() { Serial.println("Connecting to AP ..."); WiFi.begin(WIFI_AP, WIFI_PASSWORD); while (WiFi.status() != WL_CONNECTED) { delay(500); Serial.print("."); } Serial.println("Connected to AP"); }void reconnect() { // Loop until we're reconnected status = WiFi.status(); if ( status != WL_CONNECTED) { WiFi.begin(WIFI_AP, WIFI_PASSWORD); while (WiFi.status() != WL_CONNECTED) { delay(500); Serial.print("."); } Serial.println("Connected to AP"); } }

Capture Dashboard

รูปถ่ายหน้า Web Broswer

รูปการทดสอบ 1

รูปการทดสอบ 2