การควบคุมเครื่องจักรอัจฉริยะโดยใช้การสื่อสารระหว่างเครื่องจักรกับเครื่องจักร M2M - Intelligence Machine Control

ขื่อ-สกุล : นายธนพล กาศักดิ์

4/4: -- คำถามท้ายบทเพื่อทดสอบความเข้าใจ

Quiz_201 – Read Modbus RTU

< รูปอุปกรณ์ที่ใช้ทดสอบ ขณะทำการทดสอบ >

< รูปอุปกรณ์ที่ใช้ทดสอบ ขณะทำการทดสอบ >

< โปรแกรมทดสอบ > #define RS485Transmit HIGH #define RS485Receive LOW #define RS485Control 4 //RS485 Direction control #define Pin_LEDMonitor 2int Wr_Index, StepConut = 0; byte Request[] = {0x01, 0x03, 0x00, 0x00, 0x00, 0x02, 0xC4, 0x0B}; byte Echo[20];void setup() { pinMode(Pin_LEDMonitor, OUTPUT); pinMode(RS485Control, OUTPUT); Serial.begin(9600); Serial2.begin(9600); digitalWrite(RS485Control, RS485Receive); Serial.println("\nStart Test MODBUS RTU"); } void loop() { Serial.print("\nTest("); Serial.print(++StepConut); Serial.print(") >>"); digitalWrite(Pin_LEDMonitor, HIGH); digitalWrite(RS485Control, RS485Transmit); delay(10); for (int i = 0; i < sizeof(Request); i++) { Serial2.write(Request[i]);} delay(10); digitalWrite(RS485Control, RS485Receive); digitalWrite(Pin_LEDMonitor, LOW); Wr_Index = 0; for (long int i = 0; i < 300000; i++) { if (Serial2.available()) Echo[Wr_Index++] = Serial2.read(); if (Wr_Index > 12) i = 999999; delayMicroseconds(10); } for (int i = 0; i < Wr_Index - 1; i++) { Serial.print(" "); if (Echo[i] < 0x10) Serial.print("0"); Serial.print(Echo[i], HEX); } float Tempp = (Echo[3] * 256 + Echo[4]) / 10.0; Serial.print(" > Tempp('C)="); Serial.print(Tempp, 2); float Humid = (Echo[5] * 256 + Echo[6]) / 10.0; Serial.print(" > Humid(%)="); Serial.print(Humid, 2); delay(5000); }

< ผลการทดสอบ >

Quiz_202 – Write Modbus RTU

< รูปอุปกรณ์ที่ใช้ทดสอบ ขณะทำการทดสอบ >

< รูปอุปกรณ์ที่ใช้ทดสอบ ขณะทำการทดสอบ >

< โปรแกรมทดสอบ > #define RS485Transmit HIGH #define RS485Receive LOW #define RS485Control 4 //RS485 Direction control #define Pin_LEDMonitor 2byte Board_ID = 0x03; byte Mdbs_Cmd = 0x06; byte H_RelayID = 0x00; byte L_RelayID = 0x03; byte Relay_On = 0x01; byte Relay_Off = 0x02; byte OnOff_Dly = 0x00; byte HByte_CRC = 00; byte LByte_CRC = 00; int StepConut = 0; byte Echo[20];void setup() { pinMode(Pin_LEDMonitor, OUTPUT); pinMode(RS485Control, OUTPUT); Serial.begin(9600); Serial2.begin(9600); digitalWrite(RS485Control, RS485Receive); Serial.println("Start Test MODBUS RTU"); }uint16_t CRC16_Update(uint16_t tempCRC, uint8_t inData) { tempCRC ^= inData; for (int i = 0; i < 8; ++i) if (tempCRC & 1) tempCRC = (tempCRC >> 1) ^ 0xA001; else tempCRC = (tempCRC >> 1); return tempCRC; }uint16_t SendByte_CRCUpdate(uint16_t tempCRC, uint8_t inData) { Serial2.write(inData); if (inData < 0x10) Serial.print("0"); Serial.print(inData, HEX); Serial.print(" "); tempCRC = CRC16_Update(tempCRC, inData); return tempCRC; } void RTU_RelayCtrl(int rly_ID, byte rly_Cmd) { uint16_t Calc_CRC = 0xffff; // the initial value H_RelayID = highByte(rly_ID); L_RelayID = lowByte(rly_ID); digitalWrite(Pin_LEDMonitor, HIGH); digitalWrite(RS485Control, RS485Transmit); delay(10); Calc_CRC = SendByte_CRCUpdate(Calc_CRC, Board_ID); Calc_CRC = SendByte_CRCUpdate(Calc_CRC, Mdbs_Cmd); Calc_CRC = SendByte_CRCUpdate(Calc_CRC, H_RelayID); Calc_CRC = SendByte_CRCUpdate(Calc_CRC, L_RelayID); Calc_CRC = SendByte_CRCUpdate(Calc_CRC, rly_Cmd); Calc_CRC = SendByte_CRCUpdate(Calc_CRC, OnOff_Dly); HByte_CRC = highByte(Calc_CRC); LByte_CRC = lowByte(Calc_CRC); Calc_CRC = SendByte_CRCUpdate(Calc_CRC, LByte_CRC); Calc_CRC = SendByte_CRCUpdate(Calc_CRC, HByte_CRC); delay(10); digitalWrite(RS485Control, RS485Receive); digitalWrite(Pin_LEDMonitor, LOW); Serial.println(); }void loop() { for (int relay = 1; relay <= 8; relay++) { RTU_RelayCtrl(relay, Relay_On); delay(3000); } for (int relay = 1; relay <= 8; relay++) { RTU_RelayCtrl(relay, Relay_Off); delay(3000); } }

< ผลการทดสอบ >

Quiz_203 – Read/Write Modbus RTU

< รูปอุปกรณ์ที่ใช้ทดสอบ ขณะทำการทดสอบ >

< รูปอุปกรณ์ที่ใช้ทดสอบ ขณะทำการทดสอบ >

< โปรแกรมทดสอบ > #include <ModbusMaster.h> #define RS485Transmit HIGH #define RS485Receive LOW #define RS485Control 4 //RS485 Direction control #define Pin_LEDMonitor 2 #define Slave_Sensor_ID 1 #define Slave_Relay8_ID 3 #define Slave_Ry4In4_ID 5 int state = 0; float CTempp, Hudmid; bool DgInput0, DgInput1, DgInput2, DgInput3; ModbusMaster node_Sensor; ModbusMaster node_Relay8; ModbusMaster node_Ry4In4; void preTransmission() { digitalWrite(RS485Control, RS485Transmit); } void postTransmission() { digitalWrite(RS485Control, RS485Receive); }void setup() { pinMode(RS485Control, OUTPUT); pinMode(Pin_LEDMonitor, OUTPUT); Serial.begin(115200); Serial2.begin(9600); postTransmission(); node_Sensor.begin(Slave_Sensor_ID, Serial2); // Modbus slave ID=1 node_Sensor.preTransmission(preTransmission); node_Sensor.postTransmission(postTransmission); node_Relay8.begin(Slave_Relay8_ID, Serial2); // Modbus slave ID=3 node_Relay8.preTransmission(preTransmission); node_Relay8.postTransmission(postTransmission); node_Ry4In4.begin(Slave_Ry4In4_ID, Serial2); // Modbus slave ID=5 node_Ry4In4.preTransmission(preTransmission); node_Ry4In4.postTransmission(postTransmission); }void ReadTemperature(void) { uint8_t result; // Toggle the coil at address (Manual Load Control) result = node_Sensor.writeSingleCoil(Slave_Sensor_ID, state); state = !state; // Read 2 registers starting at 0x0000) result = node_Sensor.readInputRegisters(0x0000, 2); // From=0, nByte=2 if (result == node_Sensor.ku8MBSuccess) { CTempp = node_Sensor.getResponseBuffer(0x00) / 10.0f; Hudmid = node_Sensor.getResponseBuffer(0x01) / 10.0f; } }void ReadDigitalInput(void) { uint8_t result; // Toggle the coil at address (Manual Load Control) result = node_Ry4In4.writeSingleCoil(Slave_Sensor_ID, state); state = !state; // Read 4 registers starting at 0x0000) result = node_Ry4In4.readDiscreteInputs(0, 4); // Start=0, nByte=4 if (result == node_Ry4In4.ku8MBSuccess) { int DgTemp = node_Ry4In4.getResponseBuffer(0x00); DgInput3 = (DgTemp >> 3) & 1; DgInput2 = (DgTemp >> 2) & 1; DgInput1 = (DgTemp >> 1) & 1; DgInput0 = (DgTemp >> 0) & 1; } }void RelayControl(int inputCase) { int rnMode = inputCase / 10; int nRelay = inputCase % 10; if (rnMode == 81) node_Relay8.writeSingleRegister(nRelay, 0x0100); // On RelayX if (rnMode == 80) node_Relay8.writeSingleRegister(nRelay, 0x0200); // Off RelayX if (rnMode == 41) node_Ry4In4.writeSingleCoil(nRelay, 0x01); // On RelayX if (rnMode == 40) node_Ry4In4.writeSingleCoil(nRelay, 0x00); // Off RelayX }void loop() { ReadTemperature(); ReadDigitalInput(); Serial.print("\n Tempp('C): "); Serial.print(CTempp, 2); Serial.print(", Humid(%): "); Serial.print(Hudmid, 2); Serial.print(", Sensor[0:3]: "); Serial.print(DgInput3); Serial.print("-"); Serial.print(DgInput2); Serial.print("-"); Serial.print(DgInput1); Serial.print("-"); Serial.print(DgInput0); if (Serial.available() > 0) { int DataInput = Serial.parseInt(); Serial.print("\n >> XYZ > X={8,4}Board Name, Y={1,0}On,Off, Z={0-8}RlyID >> "); Serial.println(DataInput); RelayControl(DataInput); } delay(2000); }

< ผลการทดสอบ >

Quiz_204 – PLC Test

< รูปอุปกรณ์ที่ใช้ทดสอบ ขณะทำการทดสอบ >

< รูปอุปกรณ์ที่ใช้ทดสอบ ขณะทำการทดสอบ >

< โปรแกรมทดสอบ >

< ผลการทดสอบ >